/**

\page tutorial-franka-pbvs Tutorial: PBVS with Panda 7-dof robot from Franka Emika

\tableofcontents

\section franka_intro Introduction

This tutorial explains how to do a position-based visual-servoing with the Panda 7-dof robot from Franka Emika equipped with an Intel Realsense SR300 camera.

The following video shows the resulting robot trajectory when the robot is achieving a position-based visual servoing over an Apriltag target.

\htmlonly

<p align="center"><iframe width="560" height="315" src="https://www.youtube.com/embed/7A5cqUEKXHg" frameborder="0" allow="autoplay; encrypted-media" allowfullscreen></iframe></p>

\endhtmlonly

\section franka_prereq Prerequisites

\subsection franka_prereq_hardware Hardware

We suppose here that you have:

- a <a href="https://www.franka.de/cart/">Panda robot</a> in its research version from <a href="https://www.franka.de/">Franka Emika</a> that will be controlled throw vpRobotFranka class.

- an Intel Realsense <a href="https://software.intel.com/en-us/realsense/sr300">SR300</a> camera attached to the robot end-effector. You could also use a <a href="https://click.intel.com/intelr-realsensetm-depth-camera-d435.html">D345</a> camera instead that is also compatible with the vpRealSense2 grabber used in this tutorial.

\subsection franka_prereq_rt_linux Setting up a real-time kernel

In order to control your robot using `libfranka`, the controller program on the workstation PC must run with real-time priority under a `PREEMPT_RT` kernel. This [tutorial](https://frankaemika.github.io/docs/installation.html#setting-up-the-real-time-kernel) shows how to proceed. We recall here the steps:

First, install the necessary dependencies:

$ sudo apt-get install build-essential bc curl ca-certificates fakeroot gnupg2 libssl-dev lsb-release

Then, you have to decide which kernel version to use. We recommend to choose the same as in the tutorial. At the time it was written, it was `4.14.12` and `4.14.12-rt10`. If you choose a different version, simply substitute the numbers. Having decided on a version, use curl to download the source files:

$ cd ~/visp_ws

$ mkdir rt-linux; cd rt-linux

$ curl -SLO https://www.kernel.org/pub/linux/kernel/v4.x/linux-4.14.12.tar.xz

$ curl -SLO https://www.kernel.org/pub/linux/kernel/projects/rt/4.14/older/patch-4.14.12-rt10.patch.xz

And decompress them with:

$ xz -d linux-4.14.12.tar.xz

$ xz -d patch-4.14.12-rt10.patch.xz

**Compiling the kernel**

Once you are sure the files were downloaded properly, you can extract the source code and apply the patch:

$ tar xf linux-4.14.12.tar

$ cd linux-4.14.12

$ patch -p1 < ../patch-4.14.12-rt10.patch

The next step is to configure your kernel:

$ make oldconfig

This opens a text-based configuration menu. When asked for the Preemption Model, choose the Fully Preemptible Kernel:

````

Preemption Model

1. No Forced Preemption (Server) (PREEMPT_NONE)

2. Voluntary Kernel Preemption (Desktop) (PREEMPT_VOLUNTARY)

3. Preemptible Kernel (Low-Latency Desktop) (PREEMPT__LL) (NEW)

4. Preemptible Kernel (Basic RT) (PREEMPT_RTB) (NEW)

> 5. Fully Preemptible Kernel (RT) (PREEMPT_RT_FULL) (NEW)

````

We recommend keeping all options at their default values. Afterwards, you are ready to compile the kernel. As this is a lengthy process, set the multi threading option -j to the number of your CPU cores:

$ fakeroot make -j4 deb-pkg

Finally, you are ready to install the newly created package. The exact names depend on your environment, but you are looking for `headers` and `images` packages without the `dbg` suffix. To install:

$ sudo dpkg -i ../linux-headers-4.14.12-rt10_*.deb ../linux-image-4.14.12-rt10_*.deb

Reboot the computer

$ sudo reboot

The version of the kernel is now `4.14.12`

$ uname -msr

Linux 4.14.12 x86_64

**Allow a user to set real-time permissions for its processes**

After the `PREEMPT_RT` kernel is installed and running, add a group named `realtime` and add the user controlling your robot to this group:

$ sudo addgroup realtime

$ sudo usermod -a -G realtime $(whoami)

Afterwards, add the following limits to the realtime group in `/etc/security/limits.conf`:

@realtime soft rtprio 99

@realtime soft priority 99

@realtime soft memlock 102400

@realtime hard rtprio 99

@realtime hard priority 99

@realtime hard memlock 102400

The limits will be applied after you log out and in again.

\subsection franka_prereq_libfranka Install Franka library

As described <a href="https://frankaemika.github.io/docs/installation.html#building-from-source">here</a>, to install the Franka library, follow the steps:

$ sudo apt install build-essential cmake git libpoco-dev libeigen3-dev

$ cd ~/visp_ws

$ git clone --recursive https://github.com/frankaemika/libfranka

$ cd libfranka

$ mkdir build

$ cd build

$ cmake .. -DCMAKE_BUILD_TYPE=Release

$ make -j4

$ sudo make install

\subsection franka_prereq_librealsense Install Realsense library

Following the <a href="https://github.com/IntelRealSense/librealsense/blob/master/doc/installation.md">tutorial</a>, we recall the main steps here:

\note Since our kernel is 4.12+ streaming Depth/IR/Color is supported and is provided out of the box. This means that the patches are not needed.

Get `librealsense` from github:

$ cd ~/visp_ws

$ git clone https://github.com/IntelRealSense/librealsense.git

$ cd librealsense

Video4Linux backend preparation:

1/ Ensure no Intel RealSense cameras are plugged in.

2/ Install openssl package required for kernel module build:

$ sudo apt-get install libssl-dev

* Install udev rules located in librealsense source directory:

$ sudo cp config/99-realsense-libusb.rules /etc/udev/rules.d/

$ sudo udevadm control --reload-rules && udevadm trigger

* Install the packages required for librealsense build:

$ sudo apt-get install libusb-1.0-0-dev pkg-config libgtk-3-dev

$ sudo apt-get install libglfw3-dev

Build and install librealsense

$ mkdir build

$ cd build

$ cmake .. -DBUILD_EXAMPLES=ON -DCMAKE_BUILD_TYPE=Release

$ make -j4

$ sudo make install

Connect your Realsense camera (we are using a SR300) and check if you are able to acquire images running:

$ ./examples/capture/rs-capture

If you are able to visualize the images, it means that you succeed in librealsense installation.

\subsection franka_prereq_target Print an Apriltag target

We provide a ready to print `36h11` tag that is 12 by 12 cm square <a href="http://visp-doc.inria.fr/download/apriltag/tag36_11_00000-120x120.pdf">[download]</a> that you may print.

If you are interested to get other tags, follow the steps described in \ref apriltag_detection_print.

\subsection franka_prereq_calib_extrinsic Calibrate extrinsic camera parameters

Follow the steps described in \ref tutorial-calibration-extrinsic in order to estimate the end-effector to camera transformation. This step is mandatory to control the robot in cartesian in the camera frame.

\subsection franka_prereq_visp_build Configure and build ViSP

Since you installed new `libfranka` and `librealsense` 3rd parties, you need to configure again ViSP with cmake in order that ViSP is able to use these libraries. To this end follow \ref install_ubuntu_visp_config. At this step you should see new `USE_FRANKA` and `USE_LIBREALSENSE2` cmake vars appearing in the CMake GUI.

Now follow the instructions for \ref install_ubuntu_visp_build.

\subsection franka_configure_ethernet Configure Ethernet

Our robot controller has by default IP `192.168.1.1`. Here we show how to configure a laptop that is connected with an Ethernet cable to the robot controller.

Edit Ethernet connections:

\image html img-netwok-connexion.png

Add a new connexion using "Add" button. Choose the default Ethernet connection type:

\image html img-ethernet-connexion-add.png

Click "Create" button in order to create a new Franka controller connection that has a static IPv4 like `192.168.1.10` and netmask `255.255.255.0`:

\image html img-franka-ethernet-edit-connection.png

Click "Save" button.

\subsection franka_connect_desk Connect to Franka desk

Select the new Ethernet Networks connection named "Franka controller". When the connection is established open a web browser like Firefox or Chromium and enter the address `https://192.168.1.1/desk`. The first time you will be warned that the connection is not secure. Click "Advanced" and "Add Exception":

\image html img-franka-firefox-exception.png

Then confirm security exception

\image html img-franka-firefox-confirm-exception.png

When connected, you may release the user-stop button and open brakes:

\image html img-franka-firefox-open-brakes.png

\section franka_pbvs Position-based visual servoing

An example of position-based visual servoing using Panda robot equipped with a Realsense camera is available in servoFrankaPBVS.cpp.

- Attach your Realsense camera to the robot end-effector

- Put an Apriltag in the camera field of view

- If not already done, follow \ref tutorial-calibration-extrinsic to estimate \f$^e{\bf M}_c\f$ the homogeneous transformation between robot end-effector and camera frame. We suppose here that the file is located in `tutorial/calibration/eMc.yaml`.

Now enter in `example/servo-franka folder` and run `servoFrankaPBVS` binary using `--eMc` to locate the file containing the \f$^e{\bf M}_c\f$ transformation. Other options are available. Using `--help` show them:

$ cd example/servo-franka

$ ./servoFrankaPBVS --help

./servoFrankaPBVS [--ip <default 192.168.1.1>] [--tag_size <marker size in meter; default 0.12>] [--eMc <eMc extrinsic file>] [--quad_decimate <decimation; default 2>] [--adaptive_gain] [--plot] [--task_sequencing] [--no-convergence-threshold] [--verbose] [--help] [-h]

Run the binary activating the plot and using a constant gain:

$ ./servoFrankaPBVS --eMc ../../tutorial/calibration/eMc.yaml --plot

Use the left mouse click to enable the robot controller, and the right click to quit the binary.

At this point the behaviour that you should observe is the following:

\image html img-franka-pbvs-start.png Legend: Example of initial position. The goal is here to bring the RGB frame attached to the tag over the yellow frame corresponding to the desired position of the tag in the camera frame.

\image html img-franka-pbvs-converge.png Legend: Example of final position reached after position-based visual servoing. In green, you can see the trajectories in the image of the tag corners and tag cog. The latest correspond to the trajectory of the projection in the image of the tag frame origin. The 3D trajectory of this frame is a straight line when the camera extrinsic parameters are well calibrated.

\image html img-franka-pbvs-converge-curves.png  Legend: Corresponding visual-features (translation and orientation of the \e cdMc homogeneous matrix corresponding to the transformation between the desired camera pose and the current one) and velocities applied to the robot in the camera frame. You can observe an exponential decrease of the visual features.

You can also activate an adaptive gain that will make the convergence faster:

$ ./servoFrankaPBVS --eMc ../../tutorial/calibration/eMc.yaml --plot --adaptive_gain

You can also start the robot with a zero velocity at the beginning introducing task sequencing option:

$ ./servoFrankaPBVS --eMc ../../tutorial/calibration/eMc.yaml --plot --task_sequencing

And finally you can activate the adaptive gain and task sequencing:

$ ./servoFrankaPBVS --eMc ../../tutorial/calibration/eMc.yaml --plot --adaptive_gain --task_sequencing

To learn more about adaptive gain and task sequencing see \ref tutorial-boost-vs.

\section franka_pbvs_next Next tutorial

You are now ready to see the \ref tutorial-franka-ibvs that shows how to implement an image-based visual servoing scheme with the Franka robot. You may also follow \ref tutorial-ibvs that will give some hints on image-based visual servoing in simulation with a free flying camera.

*/